Submitted By:

Shahroz Ahmed-2025-CE-115

Assignment:

Lab No. 10

Submitted To:

Sir Mehmood

For the fulfillment of,

Programming Fundamental Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

Explanation of work on proteus

Exercise no.1:

- In Proteus, resistors of required values were selected from the library. A DC voltage source (10V) was used. A DC ammeter and DC voltmeter (multimeter) were used for measurements along with ground.

DC Source (10V).

DC Ammeter.
Resistor.
DC Voltmeter (Multimeter).

- All components were selected from the “Pick Devices” menu in Proteus. The DC source, resistors, ground, and measuring instruments were placed on the workspace.

- The circuit was connected as per the given diagram. The resistors were connected in the required network configuration, and the load resistance was connected across the output terminals.

- The DC ammeter was connected in series with the load resistance. The circuit was completed, and current through the load was observed.

- To find Vth, the load resistance was removed from the circuit.
A voltmeter was connected across the open terminals, and the open circuit voltage was measured as 5V.

- To find Rth, the DC source was replaced with a short circuit.
A multimeter (ohmmeter mode) was connected across the open terminals, and the equivalent resistance was measured as 495Ω.

- A new circuit was constructed using the obtained values:

- Thevenin Voltage = 5V

- Thevenin Resistance = 495Ω

The load resistance was connected in series with Rth and the DC source.

- After completing all connections, the simulation was started using the Run button in Proteus.

- The current through the load in the equivalent circuit was measured and compared with the original circuit.
It was verified that both currents are equal, thus confirming Thevenin’s Theorem.